Hadley

Navigating the Bookshelf

Presented by Vikki Vaughan

Vikki: Hi, I'm Vikki Vaughn and today we'll explore using the bookshelf in the NLS standard digital Talking Book player.

If you're unsure about which player you're using, there's an easy way to distinguish between the standard and advanced players. You'll want to locate the round speaker on the front of your player. If there are five keys below the speaker, labeled Info, Previous, Menu, Next, and Mark, you have the advanced player. The standard player does not have these five buttons. If you have the NLS advanced Digital Talking Book player, check out Hadley's version of this workshop especially designed for your player.

In another workshop in this series, we examined all of the buttons on our player and the function of each. Today we'll jump right into using several of these buttons to move between books stored on our portable storage devices.

Let's start off by inserting our storage device into our player. For today's demonstration we'll use an NLS cartridge, We use the same technique we learned today whether we're using an NLS cartridge, or USB or flash drive. Now I've downloaded several books from the bar catalog unto my NLS cartridge. If you'd like more information about downloading your favorite book or magazines, make sure you visit our Bard instructional series. Let's power up our player by pressing the power button, our player will beep and then automatically start playing from where it last left off.

NLS Player: Player on, three books. Diabetes and heart healthy cookbook by American Heart Association, American Diabetes Association, db 62692.

Vikki: Let's press our play stop button to stop playback. Well we know that our play stop button is the one we use when we wish to start or stop playback. But it also has an another important function, it allows us to open our bookshelf where we navigate between books on our storage device. Imagine we're standing in front of a bookshelf filled with books, we've placed our hands on one of the shelves. When we move our fingers to the right, we land on a book. When we move again to the right, we land on another book. If we move our hands to the left, we land on a book and again to the left, we find yet another book.

Our virtual bookshelf is much like this imaginary bookshelf, Our play stop button takes us to the bookshelf, and our fast forward and rewind buttons move us back and forth through our shelf. Let's activate our bookshelf, we'll hold down our play stop button until we hear a beep and our player announces bookshelf.

NLS Player: Bookshelf, three books, book one. Moby Dick, or the whale.

Vikki: Once we hear the announcement, we lift our finger from the button. After the announcement we hear that we currently have three books on our bookshelf, and the focus is currently on the first book, "Moby Dick." All of this information is really helpful, We know we're in our bookshelf, there are three books there and we currently are on the first book. That means we have more books to explore.

Let's think about our imaginary bookshelf we discussed earlier. If we wish to move to our next book on the shelf, we move to the right. With our players bookshelf, we press our fast forward button to move to our right. It's conveniently located to the right of our play stop button. Let's press it now.

NLS Player: Two, "Diabetes and heart healthy cookbook."

Vikki: We heard a beep the number two and the title of our next book. Let's press the fast forward ,one more time to hear the title of our third book.

NLS Player: Three, "The journeyer."

Vikki: Okay, just for kicks, let's press the fast forward button one more time.

NLS Player: Three, "The Journeyer."

Vikki: We heard the button beep followed by two quick beeps and the title of our third book. Those two quick beeps tell us that we've reached the end of our book list, the player repeated the title of our third book. Now I bet it's no surprise that when we wish to move back through our shelf, we move to the left. Let's press our rewind button one time.

NLS Player: Two, "Diabetes and heart healthy cookbook."

Vikki: We heard that we've moved back to our second book, and we heard its title, another press of the rewind button, and our player announced the number one and the title of our first book.

NLS Player: One, "Moby Dick or the whale."

Vikki: Let's press the rewind button one more time.

NLS Player: One, "Moby Dick or the whale."

Vikki: Just like with our fast forward button, we heard the double beep which tells us we've reached the top of our book list. Okay, we've moved back and forth through our bookshelf. Now it's time to select our book, Once we hear the title we wish to open, all we need to do is press and release the play stop button. Let's move forward to our second book and do just that,

NLS Player: Two, "Diabetes and Hearth Healthy Cookbook."

Vikki: When we pressed our fast forward button, we heard the familiar beat the book number and the title of our book. We know we're in the right place, so let's press and release the play stop button.

NLS Player: Copyright 2004 by the American Heart Association and the American Diabetes Association Incorporated, read by Barbara Panolini.

Vikki: We heard a beep in the title of our book, our player automatically starts playing our book for us. I pressed our play stop button once again to stop playback. If we've already started our book, our player will resume playback where we last left off. Congratulations, you've just used the bookshelf on the standard NLS player. You can now easily navigate between books, magazines, even music files on your portable storage devices. Today we explored the bookshelf in the standard NLS talking about player. I'm Vikki Vaughn and I thank you for joining me.